PCB manufacturing research – JLCPCB vs PCBWay

This report compares PCB fabrication options between JLCPCB and PCBWay for the CubeSat Electrical Power System (EPS) prototype. The goal is to identify a reliable and cost-effective manufacturer while maintaining high-quality electrical and mechanical performance.

| Parameter | JLCpcb | PCBway | Notes |
| --- | --- | --- | --- |
| Base Price (4 Layers) | 7$ | 23$ | JLCPCB offers frequent 4-layer promotions for small prototypes |
| Base Price (2 Layers) | More expensive (via tenting adds ~20$) | Cheaper but fewer options | JLCPCB charges extra for via tenting on 2-layer boards |
| Size | 100 × 100 mm | 100 × 100 mm | Standard prototype board size |
| Quantity | 5 pcs | 5 pcs | Small prototype batch |
| Thickness | 1.6 mm | 1.6 mm | Standard PCB thickness |
| Copper weight | 1 oz (inner & outer) | 1 oz | Sufficient for low-to-medium current levels |
| Min Trace width | 3.5 mil (0.09 mm) | 4 mil (0.10 mm) | Higher precision allows denser routing |
| Solder mask | Green (default) | Green (default) | Cheapest and easiest to inspect visually |
| Surface Finish | ENIG (nickel and gold plated) | HASL or ENIG | ENIG is preferred for EPS: better thermal, corrosion, and electrical characteristics |
| Via covering | Tented (default on 4-layer | Tented or Plugged (extra cost) | 4-layer tenting included by default at JLCPCB |
| Build time | Standard 3-4 days | Standard 3-4 days | Expedited builds available but increase cost |

Conclusion:

After careful comparison, JLCPCB is the preferred manufacturer for the CubeSat EPS prototype, it offers 4-layers PCB’s for only 7$, compared to 23$ at PCBway. JLCPCB included tented vias by default and provides reliable ENIG surface finishing, ensuring precision, corrosion resistance and superior solderability. Ideal for circuits involving

ADC readings, current monitoring and fine-pitch SMDs. PCBWay, while reputable, is significantly more expensive for equivalent builds.